SR-DH series

DH60/DH100/DH120(-R/-W）Gen4

# Intelligent wireless LED dimming type

All-in-one constant current solar street light controller

SR-DH120-R	SR-DH100-R	SR-DH60-R

010      101

101      010

010      101

PWM

Features

Power

100%

10%

100% 30% soc

- Whole new 2.4G wireless/infrared remote control design allows manual modification of controller parameters and reading of system information.

- Full digital high precision constant current control, achieving the maximum efficiency of 96%.

- Human motion infrared/microwave sensing function, with sensing delay time settable.

- 9-Period light dimming and pre-dawn lighting design, working hours settable from 0 hours to 15 hours, power settable from 0% to 100%.

- Both lead-acid battery and lithium battery are applicable, operating parameters can be set by remote control.

- A variety of intelligent power modes are available for choice, with load power adjustable automatically according to the battery level.

- Very low sleep current for long-distance transportation and storage.

- System status recording allows recording of up to 7 days of system status, realizing full monitoring of the system.

- Metal shell, IP67 waterproof rating, applicable to a variety of harsh environments.

- High precision digital step-up constant current control algorithm provides high efficiency and high constant current accuracy, extending the service life of battery.

- Battery charge and discharge high and low temperature protections guarantee that when the temperature exceeds a certain temperature, the number of loads will be reduced or the load will be turned off.

- Battery reverse polarity protection, LED short circuit and open circuit protection, etc. are provided for  full protection of the system.

www.szshuori.com

Exterior

Infrared receiving

Battery indicator

Load indicator

Charge indicator

Wiring Diagram

IR remote

Status

Model：SR-DH120-R

Working Volatage：12/24V Auto Max. Charging Current：20A

Max. LED Power：60W/12V；120W/24V

Gen4 Intelligent PWM Solar Charge Controller With Step-up LED Driver

www.szshuori.com

State Indicators

DH120/100 dimensions:

Overall dimensions: 82*100*20mm Mounting dimensions: 86*75mm Mounting hole diameter: φ3.5mm

75

Φ3.5

82

DH60 dimensions：

Overall dimensions: 82*58*20mm Mounting dimensions: 43*75mm Mounting hole diameter: φ3.5mm

75

www.szshuori.com

Φ3.5

58

43

20

| LED indicator | Indication content | Status | Function | Remote control system status |
| --- | --- | --- | --- | --- |
|  | Solar panel indication | Steady on | Solar panel voltage is higher than light control voltage | Idle |
|  |  | Off | Solar panel voltage is lower than light control voltage | Idle |
|  |  | Slow flash | In charging | Charging |
|  |  | Double flash | Battery is fully charged | Fully charged |
|  |  | Quick flash | Lithium battery BMS protection Battery overvoltage;PV panel overvoltage Over temperature (ambient temperature) | E-BMS Battery overvoltage PV panel overvoltage Over temperature |
|  | Battery indication | Steady on | Battery works properly | Idle |
|  |  | Off | Battery is not connected or lithium battery protection board overdischarge protection | / |
|  |  | Quick flash | Battery over-discharge | Over discharge |
|  |  | Single flash (One flash per 10 seconds) | Overdischarge, sleep | / |
|  | Load indication | Steady on | Load is turned on | Discharging |
|  |  | Off | Load is turned off | Idle |
|  |  | Slow flash | Load is open circuited | Open circuit |
|  |  | Quick flash | Load is short circuited | Short circuit |

82

100

86

20

Installation Dimensions

Parameters

www.szshuori.com

| Items |  | Values | Adjustable |  | Default |
| --- | --- | --- | --- | --- | --- |
| Model | DH60 | DH100 | DH120 |  |  |
| Remote control type | 2.4G wireless remote control: -W Infrared remote control: -R |  |  |  |  |
| System voltage | 12V/24V |  |  |  |  |
| Zero load loss | -R ：<10mA/12V；<15mA/24V -W：<32mA/12V；<38mA/24V |  |  |  |  |
| Sleep loss | <0.8MA/12V；<8MA/24V |  |  |  |  |
| Maximum charge current | 10A | 15A | 20A |  |  |
| Solar input voltage | ≤ 55V |  |  |  |  |
| Typical efficiency of constant current source | 90% ～ 96% |  |  |  |  |
| Output power | 40W/12V 60W/24V | 50W/12V 100W/24V | 60W/12V 120W/24V |  |  |
| Output current | 50mA～2000mA | 50mA～3300mA | 50mA～4000mA | √ | 330mA |
| Over voltage | Lead-acid battery: 16V; lithium battery: charge voltage +2V |  |  |  |  |
| Equalizing charge voltage | Lead-acid battery: 14.6V; lithium battery: no equalizing charge |  |  |  |  |
| Equalizing charge interval | 30 days |  |  |  |  |
| Boost charge voltage (lead-acid) Charge voltage (lithium) | 9.00V ～17.00V, settable |  |  | √ | 12.45V |
| Floating charge voltage (lead-acid) Charge return voltage (lithium) | 9.00V ～17.00V, settable |  |  | √ | 12.00V |
| Over discharge voltage | 9.00V ～17.00V, settable |  |  | √ | 09.20V |
| Over discharge return voltage | 9.00V ～17.00V, settable |  |  | √ | 10.20V |
| Temperature compensation | Lead-acid battery: -3.0mV/℃/2V; lithium battery: no compensation |  |  |  |  |
| Current accuracy | < 3% |  |  |  |  |
| Light control voltage | 3V ～ 11V |  |  | √ | 5V |
| Light control delay | 5s～60s/2min～60min |  |  | √ | 10s |
| Operating temperature | -35℃ ～ +65℃ |  |  |  |  |
| IP rating | IP67 |  |  |  |  |
| Weight | 150g | 280g | 280g |  |  |